Programming pins is SPI

TX/RX for seriall pins

Use 5v

Connect Arduino

Open ArduinoISP

Burn bootloader , Arduino as isp

Then upload using programmer for rcode

- Upload Arduino as ISP to Arduino Uno

- Connect Arduino to Atmega (RST, SCK, MISO, MOSI) -> (10, 13, 12 , 11)

- New Sketch -> tools -> programmer = Arduino as isp

  - Tools - > burn bootloader

- Get normal sketch

- Sketch -> upload using programmer